Below Freezing Power Loss Procedures

Please visit our Facebook page for an instructional video on this procedure

**Follow instructions if equipment ABOVE or AT grade (pool equipment is level with, or higher than the pool)**

***IF POOL EQUIPMENT IS BELOW GRADE CONTACT B&B POOLS SERVICE DEPARTMENT FOR FURTHER INFORMATION***

Pools with Automation Panels & Cartridge Filters

Jandy Control System w/Cartridge Filter

- Turn off breaker to pool equipment

- Remove drain plugs from pool pump (2x plugs on side of pump)

- Remove drain cap from pool filter (1x drain cap on bottom)

- Loosen unions on salt cell (3x unions)

- Remove drain plug from pool heater if applicable (1x plug near plumbing)

- Empty skimmer & pump baskets

Pentair Control System w/Cartridge Filter

- Turn off breaker to pool equipment

- Remove drain plugs from pool pump (2x plugs on side of pump)

- Remove drain plug from booster pump if applicable (1x in front of pump, 1 on side)

- Remove drain cap from pool filter (1x drain cap underneath plumbing in front – will require an adjustable wrench or 9/16th socket wrench)

- Loosen unions on salt cell (2x unions)

- Empty skimmer & pump baskets

To restart equipment after power is restored

- Re-insert all drain plugs/caps

- Open air relief on top of filter

- Remove pump lid & fill basket area with water before putting lid back on

- Turn breaker back on

Pools with other systems

Pools with SAND or DE Filters

- Follow above procedures for winterizing pool pump, booster pump, heater, salt cell

- Remove drain cap/drain plug from bottom of filter tank

To restart equipment after power is restored

- Re-insert all drain plugs/caps

- Remove pump lid & fill basket area with water before putting lid back on

- Turn breaker back on