Regal Stop Yellow

- Adjust chemical balance to the following:

  - pH: 7.8 or higher

  - Alkalinity: 80-120

  - CYA: 25-75

  - Chlorine: 2.0-2.5

- Adjust valves so circulation comes from Main Drain

- Shake product well to break up clumps

- Add _________ bags Regal Stop Yellow directly to pool

  - Spread half along the perimeter of the pool, starting where algae is thickest. Concentrate the balance directly over the problem areas

- Wait 5 minutes

- Add _________ lbs. Super Shockwave

- Wait 12 hours

- Add _________ lbs. Super Shockwave

- Wait 12 hours

- Add _________ lbs. Super Shockwave

- Wait 24 hours

- Retest pool water

- Dosage:

  - 1 bag Regal Stop Yellow per 10k Gallons

  - 2lb Super Shockwave per bag of Regal Stop Yellow

- Example: 20k Gallon Pool – 2 Bags Stop yellow, 12lb Super Shockwave

  - 2 Bags Regal Stop Yellow

  - 4lb Super Shockwave initially, 4lb at 12 hours, 4lb at 24 hours